DEMERARA SUGAR TECHNICAL SPECIFICATION

1. DESCRIPTION

Demerara sugar is vegetable origin, consisting primarily of sugarcane sucrose, with an appearance of solid with well defined crystals, clear brow color.

2. COMPOSITION

Basic composition of Demerara sugar is sucrose with a minimum concentration of 98.5%, minerals salts concentration of 0.20% and other components 0.23%.

3. TESTS

| SENSORY ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Aspect | Proper | Proper | n.a |
| Color | Light brown | -- |  |
| Smell | Proper | Proper |  |
| Flavor | Characteristic sweet | Characteristic sweet |  |

| PHYSICOCHEMICAL ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Polarization | 98,5 | -- | ºZ |
| Icumsa | -- | 2.500 | UI |
| Humidity | -- | 0,10 | % |
| Ash | -- | 0,20 | % |
| Acid Insoluble Ash | -- | 1,5 | % |
| Magnetizable Particles | -- | 5 | mg/Kg |
| Reducing Sugar | -- | 0,120 | % |
| Sucrose | 98,50 | -- | % |
| Dextran | -- | 120 | ppm |
| Insoluble residues | -- | 10 | ppm |

| MICROBIOLOGICAL ANALYSIS |  |  |  |  |
| --- | --- | --- | --- | --- |
| PARAMETERS |  | PATTERN |  | UNITY |
|  |  | MIN | MAX |  |
| Coliforms at 45ºC / g |  | -- | Nill | NMP/g |
| Molds and Yeasts |  | -- | 50 | UFC/g |
| Salmonella |  | -- | Nill | -- |
| Bacillus cereus |  | -- | Nill | -- |
| Thermophiles | Not Prod. H2S | -- | 3 | Nº(+) |
|  | Prod. H2S | -- | 5 | Esp/10g |
|  | Prod. Flat-sour | -- | 50 | Esp/g |
| Total Aerobic Mesophiles |  | -- | 50 | UFC/g |

| HEAVY METAL ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Lead | -- | 0,10 | Mg/kg |
| Arsenic | -- | 0,10 | Mg/kg |
| Antimony | -- | 2 | ppm |
| Arsenic | -- | 010 | ppm |
| Cadmium | -- | 1 | ppm |
| Copper | -- | 30 | ppm |
| Lead | -- | 0,10 | ppm |
| Chrome | -- | 0,10 | ppm |
| Tin | -- | 250 | ppm |
| Mercury | -- | 0,01 | ppm |
| Nickel | -- | 5 | ppm |

| MICROSCOPIC ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Animal Fragments | -- | Nill | -- |
| Metallic Fragments | -- | Nill | -- |
| Insect Fragments | -- | Nill | -- |
| Synthetic Fragments | -- | Nill | -- |
| Mineral Fragments | -- | Nill | -- |
| Vegetable Fragments | -- | Nill | -- |
| Strange Matter Fragments | -- | Nill | -- |
| Parasites and Larvae | -- | Nill | -- |
| Sand | -- | Nill | -- |
| Mites | -- | 5 | -- |

4. NUTRITIONAL INFORMATION:

| QUANTITY PER PORTION |  |  |
| --- | --- | --- |
| Portion of 5gr (1 Teaspoon) |  | % DV |
| Energetic value | 20 k cal = 84 kJ | 1 |
| Carbohydrates | 5gr | 1 |
| Iron | 2 mg | 1 |
| Thiamine | 0,5 mg | 42 |
| Riboflavin | 0,3 mg | 23 |
| Does not contain significant amounts of protein, total fat, saturated fat, trans fat, dietary fiber and sodium. |  |  |

% Daily Values based on a 2,000kcal or 8,400kJ diet. Your daily values may be bigger or smaller depending on your energy needs.