LIQUID SUGAR

1. DESCRIPTION

a) Liquid sugar is a natural sucrose sweetener presented in liquid form in a odorless, clear and crystalline solution, obtained by dissolving sugar in water with further purification and discoloration.

b) Liquid Sugar is produced from the dissolution of crystal sugar, free of fermentation, free of matter earth, parasites and animal or vegetable debris.

- COMPOSITION:

C12H22O11 (Sucrose molecule) + H2O (water molecule)

Sucrose also known as sugar is a type of glycide, formed by a molecule of glucose and one of fructose, produced by the sugar cane when performing the photosynthesis process.

- PHYSICAL AND CHEMICAL CHARACTERISTICS

| CRITERIA | SPECIFICATION | DESCIPTION |
| --- | --- | --- |
| TSD | 67 ± 0,3 | Brix (%) |
| ASH | 0,05 | (%) máx |
| ICUMSA COLOR | 100 | (UI) máx |
| pH | 6,50 a 7,00 |  |
| Max Turbidity (NTU) | 2,00 |  |
| Odor after acidification |  | No abnormal odoR |
| Flakes (Alcoholic Test) |  | absent |
| Total sugar | 99,90 | (%) |
| Gravimetric Sediments | 7 | (mg/kg) máx |
| Invertes Sugar (AR) | 0,30 | % (p/p) máx |
| Sulfite | 6,00 | (mg/kg) máx |
| Acid Flake (ten day test) |  | Negative |
| Quaternary Compounds of Ammonium | 2 | (ppm) máx. |
| Temperature Max.Charging | 35,0 | (ºC) AT |
| Appearance: |  | Clear, free from turbidity and visible particles,such as coal and foreign materials. |

- MICROBIOLOGICAL CONTAMINANTS

| CRITERIA | SPECIFICATION | DESCIPTION |
| --- | --- | --- |
| Yeast and Moldes | 50 | Max (UFC/10ml) |
| Total coliforms | Nill | Max (UFC/10ml) |
| Fecal coliforms | Nill | Max (UFC/10ml) |
| Total Mesophiles | 200 | Max (UFC/10ml) |

Analytical method: 					  filtration

- CHEMICAL / INORGANIC CONTAMINANTS

| CRITERIA | SPECIFICATION | DESCIPTION |
| --- | --- | --- |
| Iron | 3,00 | Max (mg/kg) |
| Copper | 1,50 | Max (mg/kg) |
| Lead | 0,10 | Max (mg/kg) |
| Arsenic | 0,10 | Max (mg/kg) |
| Mercury | 0,01 | Max (mg/kg) |

Brazilian Ministery of Agriculture Law  nº55871/65 / Resolution 42/13 (Anvisa)

5. POTENCIAL USE

• It is the most practical sweetener for most industrial food applications like ice cream, pasta, beverages, juices, bakeries, pharmaceutical industries or in chemical processes where uses sucrose (detergents, paints, plastics, glues, etc ...). Quite applied where the absence of color is an essential factor and is the ideal product for replacing solid sugars in processes where there is dilution or addition of water at some stage.

• Due to its characteristics, it is easy to use and always guarantees a superior standard and uniformity in syrup and syrup production.

• Avoids costly dilution, storage and transport processes for solid sugars and ensures

the total exemption of crystals in the final product.

• Does not interfere with production with problems such as black dots.

• Eliminates the possibility of losses and facilitates storage conditions.

• Reduces water consumption.

• Reduces or minimizes the production of industrial waste from the filtration and

deodorization of syrup or syrup.

• Due to its “just-in-time” consumption characteristics, it provides gains by reducing storage of raw material.

• It allows for a high degree of automation of the production line to be easily achieved.

- TRANSPORTATION CONTENT

Felxitank with 24 mt